Odometery with the rotation sensors.

What are rotation sensors?

The VEX V5 Rotational Sensor is a high-resolution, absolute and relative position sensor designed to track angular motion very accurately. Here's a breakdown of how it works, what it measures, and why it’s useful, especially in odometry and motion tracking.

🔧 How It Works

The VEX V5 Rotational Sensor uses an internal magnetic encoder to detect rotation. Here's the process:

1. Magnetic Sensing (Hall Effect)

- The sensor contains a magnet attached to a rotating shaft and a Hall-effect sensor that detects the orientation of the magnetic field.

- This allows the sensor to determine the absolute angular position of the shaft within one revolution (0–360°).

2. Absolute and Relative Tracking

- Absolute Mode: Tracks the current position of the shaft within 0–360°.

- Relative Mode: Tracks how many full rotations the shaft has turned, accumulating over time. This is used to calculate total distance or angle turned, even across multiple revolutions.

3. Digital Output to V5 Brain

- The sensor connects via a smart port to the VEX V5 Brain.

- It reports:

  - Angle (°) — precise position within a single revolution.

  - Rotation (rotations or degrees) — total accumulated angle since the sensor was reset.

  - Velocity (deg/sec or RPM) — current speed of rotation.

📐 Key Features

| Feature | Value |
| --- | --- |
| Resolution | 0.1° (3,600 counts per revolution) |
| Measurement Range | ±2.1 billion degrees (theoretical) |
| Accuracy | Very high — suitable for odometry |
| Interface | Smart Port (VEX V5 system) |
| Mounting | 3-hole C-channel compatible |
| Type | Magnetic (non-contact, no mechanical wear) |

🔄 Use Cases

✅ Odometry / Position Tracking

- Mount the sensor on a passive (tracking) wheel to track linear robot motion.

- Accurately detect forward/backward and side-to-side translation using three sensors (standard 3-wheel odometry).

✅ Joint / Arm Positioning

- Precisely measure the rotation of arms, lifts, or turrets.

- Maintain positions using PID control based on rotational feedback.

🧠 How It Compares

| Sensor | Encoder Type | Absolute Position? | Continuous Rotation Tracking? | Use in Odometry |
| --- | --- | --- | --- | --- |
| Rotational Sensor | Magnetic Encoder | ✅ Yes | ✅ Yes | ✅ Excellent |
| Motor Encoder | Internal Optical | ❌ No | ✅ Yes (limited precision) | ❌ Not ideal |
| Potentiometer | Analog Resistance | ✅ Yes | ❌ No (limited to ~265°) | ❌ Not suitable |

🛠️ Programming Note

In VEXcode V5 (Blocks or Text), you can use:

RotationSensor.position(degrees);

RotationSensor.rotation(degrees);  // Total accumulated rotation

RotationSensor.velocity(rpm);

RotationSensor.resetRotation();

In PROS:

Rotation my_rotation_sensor(PORT);

double angle = my_rotation_sensor.get_angle();  // in thousandths of a degree

double position = my_rotation_sensor.get_position();  // in degrees

Links

Odometry - GeeksforGeeks

Intro to Odometry, Part 1

tropical's site

Odometry tutorial · GitHub

Odometry Code? - Programming Support / VEXcode V5 Tech Support - VEX Forum